# Insta360: Avoiding Glitchy Audio in Clips

Checklist to prevent the "one clip with broken audio" problem. Most cases are caused by a momentary SD card slowdown, a firmware bug, or the wind-noise-reduction processing at export.

# Card Habits (biggest impact)

Format the card in the camera after every offload. Never free space by deleting individual clips — that fragments the card and causes mid-recording stalls.

Keep the card under ~80% full. A nearly full card has no scratch space for background housekeeping, so pauses get longer and more frequent.

Use a genuine V30 / A2 card. SanDisk Extreme Pro, Samsung Pro Plus, or Insta360-branded, 128 GB+. Buy from a real retailer, not marketplace third-party sellers (counterfeits pass short tests but fail on sustained writes).

Replace the card if bad clips repeat. One bad clip = the card probably hiccuped. Two or three corrupt clips on the same card = retire it.

# Camera Habits

Keep firmware updated. Insta360 acknowledges intermittent no-audio / abnormal-noise bugs that are fixed via firmware updates.

Manage heat on long sessions. Power the camera off for a minute or two between long takes on hot days — heat slows the SD card and triggers dropouts.

Check the mic ports. Make sure they are clear of water, dust, or your mounting grip before recording.

# Workflow / Recovery

Broken audio after export? Check the raw .insv first. If the raw file's audio is clean, the glitch was added by the AI wind-noise processing — re-export with Wind Noise Reduction off and the clip is recoverable.

Unfinalized file (battery died, card pulled)? Use the official Insta360 File Repair tool before writing the clip off.

Verify a suspect card once. Run H2testw (Windows) — if sustained write speed is under ~35 MB/s, the card is the culprit.

Rule of thumb: one bad clip = the card hiccuped; recurring bad clips on the same card = replace the card.